🌿 TECHNICAL SPECIFICATION Smart Agriculture IoT System ESP32 Gateway + Zigbee IoT Nodes ¦ ESP-IDF Native ¦ PRO Industrial Edition Document ID: SAGRI-SPEC-001 Revision: 1.0 Status: RELEASED Classification: Proprietary — Internal Engineering Use

# 1. Project Overview

## 1.1  Executive Summary

This document defines the complete technical specification for the Smart Agriculture IoT System (SAGRI), an industrial-grade wireless sensor-actuator network targeting precision agriculture applications including greenhouse automation, open-field irrigation management, soil health monitoring, and crop microclimate control.

The system is built on the Espressif ESP-IDF native framework exclusively — no Arduino or third-party RTOS abstractions. All firmware is production-grade: fully tested, fault-tolerant, and deployment-ready for real-world farm environments.

## 1.2  System at a Glance

| Parameter | Value |
| --- | --- |
| System Name | SAGRI — Smart Agriculture IoT Platform |
| Hardware Platform | ESP32-S3 (GW) + ESP32-H2 (Field Nodes) |
| Firmware Framework | ESP-IDF v5.2+ (idf.py native toolchain) |
| Wireless Protocol | Zigbee 3.0 (IEEE 802.15.4) + Wi-Fi 4 (GW) |
| Cloud Protocol | MQTT 3.1.1 over TLS 1.3 |
| Maximum Nodes | 250 per Coordinator (Zigbee spec max) |
| Operating Temp | -20 °C to +70 °C |
| Node Battery Life | > 2 years (3.6 V Li-SOCl₂ 4000 mAh) |
| Security | Secure Boot v2, Flash Encryption, Zigbee AES-128 |
| OTA Updates | GW: HTTPS; Nodes: Zigbee OTA Cluster 0x0019 |

# 2. System Architecture

## 2.1  High-Level Block Diagram (ASCII)

┌──────────────────────────────────────────────────────────────────┐

│                     CLOUD / SCADA LAYER                         │

│   MQTT Broker (TLS)   REST API Clients   OTA Update Server      │

└───────────────────────────┬──────────────────────────────────────┘

│ Wi-Fi / Ethernet (TLS 1.3)

┌───────────────────────────▼──────────────────────────────────────┐

│              ESP32-S3  GATEWAY  (Zigbee Coordinator)            │

│  ┌──────────────┐  ┌──────────────┐  ┌──────────────────────┐  │

│  │ WiFi Manager │  │ MQTT Client  │  │ REST API :8080       │  │

│  │ STA+Reconnect│  │ TLS Pub/Sub  │  │ /nodes /sensors /ota │  │

│  └──────────────┘  └──────────────┘  └──────────────────────┘  │

│  ┌──────────────┐  ┌──────────────┐  ┌──────────────────────┐  │

│  │ ZB Coord     │  │ OTA Manager  │  │ Cloud Buffer SPIFFS  │  │

│  │ Network Form │  │ GW+Node OTA  │  │ 72h offline ring buf │  │

│  └──────────────┘  └──────────────┘  └──────────────────────┘  │

└──────────────────────────┬───────────────────────────────────────┘

│ Zigbee 3.0  (IEEE 802.15.4)

┌──────────────────┼──────────────────┐

▼                  ▼                  ▼

┌──────────────┐  ┌──────────────┐  ┌──────────────┐

│ Field Node 1 │  │ Field Node 2 │  │ Field Node N │

│  ESP32-H2    │  │  ESP32-H2    │  │  ESP32-H2    │

│  Sensors:    │  │  Sensors:    │  │  Sensors:    │

│  DHT22/SHT40 │  │  DHT22/SHT40 │  │  DHT22/SHT40 │

│  Soil Moist  │  │  Soil Moist  │  │  NPK RS-485  │

│  NPK RS-485  │  │  Rain Gauge  │  │  BH1750 Lux  │

│  BH1750/SCD40│  │  BH1750/SCD40│  │  SCD40 CO2   │

│  Actuators:  │  │  Actuators:  │  │  Actuators:  │

│  Valve/Pump  │  │  Valve/Pump  │  │  Fan/PWM LED │

└──────────────┘  └──────────────┘  └──────────────┘

[Deep-sleep ED]   [Deep-sleep ED]   [Deep-sleep ED]

## 2.2  Data-Flow Description

- Field Node wakes from deep-sleep, acquires all sensor readings, applies calibration offsets (NVS)

- Processed data packaged into AGRI_CLUSTER attributes, transmitted to Gateway via Zigbee ZCL reporting

- Gateway receives Zigbee frames, decodes attribute report, maps to agri_sensor_data_t struct

- Data JSON-encoded per telemetry schema and published to MQTT topic agri/{farm}/{node}/telemetry

- If Wi-Fi/MQTT offline: data written to SPIFFS ring-buffer (max 72 h); replayed on reconnect

- Cloud sends command to MQTT topic agri/{farm}/{node}/cmd; GW forwards via Zigbee On/Off or Level cluster

- Field Node receives actuator command, executes relay/PWM control, sends acknowledgement cluster

# 3. Hardware Specifications

## 3.1  Gateway Board — ESP32-S3

| Attribute | Specification |
| --- | --- |
| MCU | ESP32-S3 — dual Xtensa LX7 @ 240 MHz |
| Flash | 8 MB SPI Flash (Quad SPI) — OTA dual partition |
| RAM | 8 MB PSRAM + 512 KB SRAM |
| Wi-Fi | 802.11 b/g/n (2.4 GHz), WPA2/WPA3 Personal |
| Zigbee | IEEE 802.15.4 (2.4 GHz), Zigbee 3.0 via co-processor or modem |
| USB | USB 2.0 OTG (programming + CDC log) |
| GPIO | 45 programmable GPIO, ADC 12-bit, I2C x2, SPI x2, UART x2 |
| Power Input | 5 V DC via USB or 7-15 V via barrel jack (LDO onboard) |
| Enclosure | DIN rail IP65 ABS, 120 × 80 × 50 mm |

## 3.2  Field Node Board — ESP32-H2

| Attribute | Specification |
| --- | --- |
| MCU | ESP32-H2 — single RISC-V @ 96 MHz |
| Flash | 4 MB SPI Flash — Zigbee OTA capable |
| RAM | 320 KB SRAM |
| Zigbee | IEEE 802.15.4 integrated transceiver, Zigbee 3.0 End Device |
| Power Supply | 3.6 V Li-SOCl₂ cell (ER26500 / AA-equiv.) via LDO + protection |
| Active Current | < 25 mA average during wake + TX cycle |
| Sleep Current | < 15 µA deep-sleep with RTC wakeup |
| UART | RS-485 half-duplex via MAX3485 transceiver for NPK sensor |
| I2C | SHT40 (addr 0x44), BH1750 (0x23), SCD40 (0x62) |
| ADC | 12-bit × 4 ch — soil moisture probes, current sense |
| GPIO | Relay driver (ULN2003), PWM fan, rain gauge pulse, LED PWM |
| Enclosure | IP67 ABS waterproof, 100 × 60 × 40 mm, M12 cable glands |

## 3.3  Sensor Specifications

| Sensor | Interface | Range / Accuracy | Update Rate |
| --- | --- | --- | --- |
| DHT22 | 1-Wire GPIO | Temp: -40–80 °C ±0.5 °C RH: 0–100% ±2% | 2 s min |
| SHT40 | I2C 0x44 | Temp: -40–125 °C ±0.2 °C RH: ±1.5% | 1 s |
| Soil Moist. | ADC 12-bit | VWC: 0–60% ±3% | On demand |
| NPK Sensor | RS-485 RTU | N/P/K: 0–1999 mg/L ±2% | 10 s |
| Rain Gauge | GPIO ISR | 0.2 mm per pulse, ±4% | Interrupt |
| BH1750 | I2C 0x23 | 1–65535 lux ±20% | 120 ms |
| SCD40 | I2C 0x62 | CO₂: 400–5000 ppm ±50 ppm | 5 s |

# 4. Zigbee Network Specification

## 4.1  Network Topology

- Star-extended mesh topology — Coordinator (GW) + optional Router nodes + End Devices

- PAN ID: 0xAGRI (configurable via NVS provisioning)

- Channel selection: automated energy scan on channels 11, 15, 20, 25 at startup

- Network key: AES-128, generated on Coordinator at formation, distributed via Trust Center

- Link key (TCLK): preconfigured unique per-device during factory provisioning

## 4.2  Custom AGRI_CLUSTER (0xFF00) — Manufacturer ID: 0x1234

| Attribute ID | Name | Type | Access | Description |
| --- | --- | --- | --- | --- |
| 0x0001 | soil_moisture_pct | U16 | R | Soil VWC × 100 (0–6000 = 0–60.00%) |
| 0x0002 | soil_temp_c | S16 | R | Soil temperature × 100 |
| 0x0003 | npk_nitrogen | U16 | R | N mg/L |
| 0x0004 | npk_phosphorus | U16 | R | P mg/L |
| 0x0005 | npk_potassium | U16 | R | K mg/L |
| 0x0006 | rain_mm_x100 | U32 | R | Cumulative rain × 100 |
| 0x0007 | lux | U32 | R | Light intensity |
| 0x0008 | co2_ppm | U16 | R | CO₂ concentration ppm |
| 0x0009 | batt_mv | U16 | R | Battery voltage mV |
| 0x000A | valve_state | Bool | R/W | Irrigation valve open/close |
| 0x000B | pump_pct | U8 | R/W | Pump PWM duty 0–100% |
| 0x000C | fan_pct | U8 | R/W | Fan duty 0–100% |
| 0x000D | led_r_pct | U8 | R/W | LED Red channel 0–100% |
| 0x000E | led_b_pct | U8 | R/W | LED Blue channel 0–100% |
| 0x000F | alarm_flags | U16 | R | Bit-field: sensor fault flags |

## 4.3  Standard Clusters Used

| Cluster | ID | Role | Used For |
| --- | --- | --- | --- |
| Basic | 0x0000 | Server | Device info, reset, SW version |
| Power Config | 0x0001 | Server | Battery percentage, voltage alert |
| Temperature | 0x0402 | Server | Ambient temp (mapped from SHT40) |
| Rel. Humidity | 0x0405 | Server | Ambient humidity |
| On/Off | 0x0006 | Server | Valve open/close commands |
| Level Control | 0x0008 | Server | Pump & fan PWM level |
| Identify | 0x0003 | Server | Commissioning identify blink |
| OTA Upgrade | 0x0019 | Client | Node firmware update via Zigbee |
| Time | 0x000A | Client | UTC time sync from coordinator |

# 5. Gateway Software Architecture

## 5.1  FreeRTOS Task Map

| Task Name | Stack | Priority | Core | Description |
| --- | --- | --- | --- | --- |
| wifi_mgr_task | 4096 | 5 | 0 | STA connect, reconnect loop, IP events |
| mqtt_pub_task | 8192 | 4 | 0 | Dequeue telemetry, publish MQTT |
| mqtt_sub_task | 4096 | 4 | 0 | Subscribe cmd topics, route to Zigbee |
| zb_main_task | 8192 | 6 | 1 | Zigbee stack main loop (ESP-ZB) |
| rest_api_task | 6144 | 3 | 0 | HTTP server + request handling |
| ota_check_task | 8192 | 2 | 0 | Periodic OTA version poll + flash |
| sntp_task | 2048 | 1 | 0 | NTP sync, RTC set |
| cloud_buffer_task | 4096 | 3 | 0 | SPIFFS ring-buffer flush on reconnect |
| sys_monitor_task | 2048 | 1 | 0 | Heap, task-WDT feeds, RSSI report |

## 5.2  Component Dependency Graph

main.c

├── nvs_config        (no deps)

├── system_monitor    (← freertos, esp_timer)

├── sntp_sync         (← esp_netif, lwip)

├── wifi_manager      (← esp_wifi, nvs_config)

│     └── [event]───► mqtt_client.connect()

├── zigbee_coordinator(← esp-zboss, nvs_config)

│     └── [cb]──────► mqtt_pub_task.enqueue()

├── mqtt_client       (← mqtt, tls, cloud_buffer)

├── rest_api          (← esp_http_server, cJSON)

├── cloud_buffer      (← spiffs, vfs)

└── ota_manager       (← esp_https_ota, esp_partition)

## 5.3  Inter-Task Communication

| Producer | Consumer | Mechanism | Payload |
| --- | --- | --- | --- |
| zigbee_coordinator | mqtt_pub_task | FreeRTOS Queue (len=64) | agri_sensor_data_t (256 B) |
| mqtt_sub_task | zigbee_coordinator | FreeRTOS Queue (len=32) | agri_cmd_t (32 B) |
| wifi_manager | mqtt_client | EventGroup bit WIFI_CONNECTED | — |
| cloud_buffer_task | mqtt_pub_task | SPIFFS file read + Queue inject | JSON string (≤4096 B) |
| ota_manager | main | EventGroup bit OTA_DONE | — |

# 6. Field Node Software Architecture

## 6.1  Wake / Sleep State Machine

DEEP_SLEEP

│  [RTC timer expires — configurable, default 30 s]

▼

BOOT_INIT     nvs_config_load(), power_manager_check_voltage()

│

▼

SENSOR_ACQUIRE  acquire all sensors in parallel (FreeRTOS tasks)

│  DHT22/SHT40 ──► I2C  │  NPK ──► UART RS-485  │  ADC soil

│  BH1750 ──► I2C       │  SCD40 ──► I2C         │  Rain count

▼

DATA_PROCESS  median filter (window=5), apply NVS calibration

▼

ZIGBEE_JOIN_CHECK  already joined? → skip;  else join network (max 60 s)

▼

ZIGBEE_REPORT  send AGRI_CLUSTER attribute report to Coordinator

│  wait ACK (max 5 s) → retry x3 → mark alarm_flag

▼

ACTUATOR_POLL  check pending actuator cmd from parent poll (if any)

▼

OTA_CHECK     Zigbee OTA cluster query (every 24 wakeups = 12 min)

▼

SLEEP_PREP    flush NVS rain counter, set RTC wakeup timer

▼

DEEP_SLEEP    esp_deep_sleep_start()

## 6.2  Power Budget

| Phase | Duration | Current | Energy (µAh) |
| --- | --- | --- | --- |
| Boot + NVS init | 300 ms | 25 mA | 2.08 |
| Sensor acquire (all) | 800 ms | 22 mA | 4.89 |
| Zigbee TX + ACK | 200 ms | 30 mA | 1.67 |
| Deep sleep (30 s cycle) | 28.7 s | 15 µA | 0.12 |
| Total per 30 s cycle | 30 s | — | 8.76 µAh |
| Daily energy | 24 h | — | 10.51 mAh/day |
| Battery life (4000 mAh) | — | — | > 380 days (> 1 year) |

# 7. Cloud Interface Specification

## 7.1  MQTT Topic Schema

| Direction | Topic Pattern | QoS | Retain | Description |
| --- | --- | --- | --- | --- |
| GW → Cloud | agri/{farm_id}/{node_id}/telemetry | 1 | No | Sensor telemetry, 30 s |
| GW → Cloud | agri/{farm_id}/{node_id}/status | 1 | Yes | Device online/offline |
| GW → Cloud | agri/{farm_id}/gw/health | 0 | No | GW heap, uptime, RSSI |
| Cloud → GW | agri/{farm_id}/{node_id}/cmd | 1 | No | Actuator commands |
| Cloud → GW | agri/{farm_id}/gw/ota | 1 | No | GW OTA trigger URL |
| Cloud → GW | agri/{farm_id}/{node_id}/ota | 1 | No | Node OTA trigger URL |
| Cloud → GW | agri/{farm_id}/config | 1 | Yes | Global config push |

## 7.2  Telemetry JSON Schema (agri/.../telemetry)

{

"schema_ver": 2,

"device_id":  "NODE-A3F9",

"fw_ver":     "1.4.2",

"ts":         "2025-06-15T08:30:00.000Z",

"loc": { "lat": 36.82, "lon": 10.17, "field": "GH-C" },

"sensors": {

"temp_c":        24.7,

"humidity_pct":  61.2,

"soil_moist_pct":38.5,

"soil_temp_c":   19.2,

"npk_n":         142,

"npk_p":         88,

"npk_k":         210,

"rain_mm":       0.0,

"lux":           18400,

"co2_ppm":       412

},

"actuators": {

"valve_open":    false,

"pump_pct":      0,

"fan_pct":       40,

"led_r_pct":     60,

"led_b_pct":     80

},

"sys": {

"batt_mv":       3681,

"rssi_dbm":      -62,

"lqi":           210,

"uptime_s":      86400,

"alarm_flags":   0

}

}

## 7.3  REST API Endpoints (Gateway :8080)

| Method | Endpoint | Auth | Description |
| --- | --- | --- | --- |
| GET | /api/v1/nodes | Bearer | List all joined Zigbee nodes |
| GET | /api/v1/nodes/{id}/sensors | Bearer | Latest sensor readings for node |
| GET | /api/v1/nodes/{id}/actuators | Bearer | Current actuator states |
| POST | /api/v1/nodes/{id}/cmd | Bearer | Send actuator command to node |
| GET | /api/v1/config | Bearer | Read NVS config (sanitised) |
| POST | /api/v1/config | Bearer | Write NVS config fields |
| POST | /api/v1/ota/gateway | Bearer | Trigger GW OTA from URL |
| POST | /api/v1/ota/node/{id} | Bearer | Trigger node Zigbee OTA |
| GET | /api/v1/health | None | GW health check (uptime, heap) |

# 8. Security Architecture

## 8.1  Security Layers

| Layer | Mechanism | Details |
| --- | --- | --- |
| Boot | Secure Boot v2 | RSA-2048 signed bootloader; chain-of-trust to app partition |
| Storage | Flash Encryption | AES-256 XTS — all flash content encrypted at rest |
| Zigbee NWK | AES-128 NWK key | Rotated on key coordinator command; no default key |
| Zigbee App | Link key per device | Unique TCLK provisioned at factory via provision.py |
| Transport | TLS 1.3 | MQTT + HTTPS OTA; mutual auth with client cert |
| API Auth | JWT Bearer token | RS256, 1 h expiry, refresh via /api/v1/auth/refresh |
| OTA Image | RSA-2048 + SHA-256 | Signature verified before boot partition switch |
| Credentials | NVS Encrypted | NVS partition encrypted separately; keys in eFuse |

🔒 eFuse Configuration (Production) • JTAG disable bit: burn to prevent debug access on production units • Secure download mode: restrict UART flashing after production provisioning • Key block 0: Secure Boot RSA-2048 public key digest • Key block 1: Flash Encryption AES-256 key (hardware generated) • WR_DIS bits: write-protect all sensitive eFuse blocks after provisioning

# 9. OTA Update Architecture

## 9.1  Gateway OTA (HTTPS)

- OTA check task polls ${OTA_SERVER}/api/v1/version?target=gw every 6 hours

- Compares semantic version in JSON response vs running FW_VERSION

- Downloads image via esp_https_ota() to standby OTA partition

- Verifies SHA-256 digest + RSA-2048 signature embedded in image

- On success: sets boot partition to new image, schedules reboot in 30 s

- On 3 consecutive boot failures: bootloader reverts to previous partition automatically

## 9.2  Field Node OTA (Zigbee OTA Cluster 0x0019)

- GW receives node OTA URL via MQTT agri/{farm}/{node}/ota topic

- GW downloads image to PSRAM / temporary SPIFFS file

- GW activates as OTA Upgrade Server on Zigbee OTA cluster

- Node queries OTA server on next wakeup — image header block transferred

- Node streams image blocks (64 B each) — resumes from last block on sleep/wake

- On complete: node verifies SHA-256, resets, new firmware boots, reports version

## 9.3  Partition Table

# partitions.csv

# Name,    Type, SubType,  Offset,   Size,     Flags

nvs,       data, nvs,      0x9000,   0x6000,

otadata,   data, ota,      0xF000,   0x2000,

phy_init,  data, phy,      0x11000,  0x1000,

ota_0,     app,  ota_0,    0x20000,  0x1E0000,

ota_1,     app,  ota_1,    0x200000, 0x1E0000,

spiffs,    data, spiffs,   0x3E0000, 0x20000,

# 10. Testing & Validation Plan

## 10.1  Unit Tests (Unity Framework via idf.py test)

- Sensor driver: inject mock I2C/UART responses, verify parsed values

- Calibration: NVS write/read roundtrip, offset/gain application

- MQTT codec: JSON encode agri_sensor_data_t, schema validate output

- Cloud buffer: fill to capacity, overflow policy, replay ordering

- OTA: mock HTTPS server, partial download + resume, bad SHA reject

## 10.2  Integration Tests

- Full Zigbee join: Coordinator + 5 ED nodes — verify all join within 60 s

- Attribute reporting: node → GW data path latency < 500 ms

- Actuator round-trip: cloud cmd → MQTT → Zigbee → relay change < 2 s

- Offline resilience: kill Wi-Fi 1 h, restore, verify all buffered data replayed

- OTA end-to-end: push new FW to GW and to 3 nodes, verify version change

## 10.3  Field Acceptance Tests

- 72-hour continuous soak: all nodes running, no watchdog reset

- Power cycle: GW and nodes powered off/on 100 times — verify self-healing rejoin

- Range test: measure LQI at 100 m open field, 50 m inside greenhouse metal structure

- Temperature stress: operate at +65 °C in enclosure (simulated summer noon)

- Battery runtime: measure actual vs predicted at typical 30 s interval

# 11. Factory Provisioning & Deployment

## 11.1  Factory Provisioning Steps

- Flash bootloader, partition table, application image via esptool.py

- Run provision.py — generates unique device_id, provisions NVS CSV with keys

- Burn Secure Boot digest + Flash Encryption key to eFuse (non-reversible)

- Burn WR_DIS bits to lock eFuse blocks

- Run factory self-test — sensor comm check, Zigbee beacon TX, MQTT ping

- Label unit with QR code encoding device_id + provisioning timestamp

## 11.2  Field Deployment Checklist

✅ Pre-Deployment Verification □ GW connected to field Wi-Fi, IP assigned, SNTP synced □ MQTT broker reachable (ping + TLS handshake verified) □ All field nodes powered, joined Zigbee network (check /api/v1/nodes) □ Sensor readings within expected range for ambient conditions □ Test actuator cycle: valve open 5 s → close (visual confirm) □ Confirm telemetry appearing in cloud dashboard □ OTA check: both GW and nodes running latest production firmware □ Alarm flag register = 0x0000 on all nodes

# 12. Key sdkconfig.defaults Settings

# Core

CONFIG_IDF_TARGET="esp32s3"

CONFIG_ESPTOOLPY_FLASHSIZE_8MB=y

CONFIG_PARTITION_TABLE_CUSTOM=y

CONFIG_PARTITION_TABLE_CUSTOM_FILENAME="partitions.csv"

# OTA

CONFIG_BOOTLOADER_APP_ROLLBACK_ENABLE=y

CONFIG_BOOTLOADER_APP_ANTI_ROLLBACK=y

# Security

CONFIG_SECURE_BOOT=y

CONFIG_SECURE_BOOT_V2_ENABLED=y

CONFIG_SECURE_FLASH_ENC_ENABLED=y

CONFIG_MBEDTLS_TLS_CLIENT=y

CONFIG_MBEDTLS_TLS_SERVER=y

# Wi-Fi

CONFIG_ESP_WIFI_STATIC_RX_BUFFER_NUM=16

CONFIG_ESP_WIFI_DYNAMIC_TX_BUFFER_NUM=32

# MQTT

CONFIG_MQTT_TRANSPORT_SSL=y

CONFIG_MQTT_BUFFER_SIZE=4096

# Logging (production)

CONFIG_LOG_DEFAULT_LEVEL_INFO=y

CONFIG_LOG_TIMESTAMP_SOURCE_RTOS=y

# FreeRTOS

CONFIG_FREERTOS_HZ=1000

CONFIG_FREERTOS_ENABLE_BACKWARD_COMPATIBILITY=n

CONFIG_ESP_TASK_WDT=y

CONFIG_ESP_TASK_WDT_TIMEOUT_S=30

# SPIFFS

CONFIG_SPIFFS_USE_MAGIC=y

CONFIG_SPIFFS_USE_MAGIC_LENGTH=y

End of Technical Specification Document — SAGRI-SPEC-001 Rev 1.0